# Lenovo ThinkSmart Hub 500
LED Ring Control on Windows

Installation guide for local control and Home Assistant integration

Purpose. This setup controls the ThinkSmart Hub 500 LED ring from Windows and optionally exposes green, red, white, and off controls to Home Assistant through MQTT.

# 1. Components to install

## Python 3.13, 64-bit

Install the 64-bit Python 3.13 release. Python 3.13 was used because the tested Python 3.15 installation did not have a compatible prebuilt hidapi package.

py -0p py -3.13 --version

## Python packages

Install the HID diagnostics and MQTT packages from an Administrator Command Prompt:

py -3.13 -m pip install --upgrade pip py -3.13 -m pip install hidapi paho-mqtt

## libusb, 64-bit

Extract the official 64-bit Windows libusb distribution and copy libusb-1.0.dll into the script directory:

C:\ThinkSmart LED\libusb-1.0.dll

## UsbDk, 64-bit

Install the 64-bit UsbDk runtime using an elevated Command Prompt:

msiexec /i "%USERPROFILE%\Downloads\UsbDk_1.0.22_x64.msi"

Restart Windows after installing this kernel-level USB component, then verify that the ThinkSmart speakers and microphone still work normally.

# 2. Verify USB access

Confirm that UsbDk is installed and that it can enumerate the Lenovo device:

where /r "C:\Program Files" UsbDkController.exe "C:\Program Files\UsbDk Runtime Library\UsbDkController.exe" -n

The relevant device should appear as:

VID: 17EF PID: A017 Serial: 201405280001

# 3. Required files

Place the completed controller and MQTT files in one directory:

C:\ThinkSmart LED\ libusb-1.0.dll thinksmart_led.py thinksmart_mqtt.py mqtt_config.json

Diagnostic scripts are not required after the installation has been validated.

# 4. Test local LED control

Run the non-destructive USB probe first:

cd /d "C:\ThinkSmart LED" py -3.13 -u thinksmart_led.py probe

Then test the validated LED states:

py -3.13 -u thinksmart_led.py green py -3.13 -u thinksmart_led.py red py -3.13 -u thinksmart_led.py white py -3.13 -u thinksmart_led.py off

# 5. Configure Home Assistant and MQTT

Enable the MQTT integration in Home Assistant.

Use an MQTT broker such as Mosquitto.

Create a dedicated MQTT account for the ThinkSmart service.

Example mqtt_config.json:

{ "host": "192.168.1.50", "port": 1883, "username": "thinksmart", "password": "REPLACE_WITH_PASSWORD", "base_topic": "thinksmart/hub500/led", "discovery_prefix": "homeassistant" }

Start the MQTT service manually for the first test:

cd /d "C:\ThinkSmart LED" py -3.13 -u thinksmart_mqtt.py

The service publishes Home Assistant MQTT discovery information and exposes the options green, red, white, and off.

# 6. Start automatically with Windows

Create a Task Scheduler task that runs at system startup with highest privileges. Use these action settings:

Program/script: C:\Users\Remoteadmin\AppData\Local\Programs\Python\Python313\python.exe Arguments: -u "C:\ThinkSmart LED\thinksmart_mqtt.py" Start in: C:\ThinkSmart LED

Trigger: At startup, optionally delayed by 30 seconds.

Run whether the user is logged on or not.

Run with highest privileges.

If the task fails, restart it after one minute.

If already running, do not start a second instance.

# 7. Architecture and safety note

Home Assistant -> MQTT -> thinksmart_mqtt.py -> thinksmart_led.py -> libusb -> UsbDk -> Lenovo USB device 17EF:A017 -> LED ring

Important: Do not replace the ThinkSmart composite USB driver with Zadig or WinUSB. The working solution uses UsbDk and libusb while retaining the normal Windows driver configuration.